# GRADES AVAILABLE - SERVO BOAT ENGINE OIL 20W-40

# APPLICATION

Servo Boat Engine Oil 20W-40 is recommended for:

- Fishing boats / trawlers

DESCRIPTION

Servo Boat Engine Oil 20W-40 is red coloured high performance diesel engine oil, which provides excellent protection against corrosive wear in all types of diesel engines. This oil is suitable for fishing trawlers during all seasons and retains high level of alkalinity during use.

# PERFORMANCE STANDARDS

Servo Boat Engine Oil 20W-40 meets the following specifications.

- API CF / CD

- E-DL 3 of IS: 13656-2002

- US Military MIL-L-2104C

- MACK T-7 test for oil thickening tendency

- Sequence III F

# PERFORMANCE BENEFITS

Servo Boat Engine Oil 20W-40 provides:

- Improved cold starting and faster engine warm up.

- Outstanding engine cleanliness even under most severe operating conditions.

- Exceptional thermal and oxidation stability to resist decomposition in super charged engines operating with high ring zone temperatures.

- Lower fuel and engine oil consumption.

- Excellent resistance to wear.

# CHARACTERISTICS

| SAE GRADE | 20W-40 |
| --- | --- |
| Kinematic Viscosity cSt @ 100C | 13.5 – 15.5 |
| Viscosity Index, Min. | 110 |
| Flash point (COC), C Min. | 200 |
| Pour Point, C Max. | (-) 21 |
| TBN mg KOH/gm | 9.5 – 12.5 |